Název investiční akce: Upgrade telefonie, (Jedná se o upgrade telefonní ústředny).

Cena projektu: 1.680.000,- Kč s DPH

Popis současný stavu :

ZZS Karlovarského kraje využívá pro hlasovou komunikaci systém OmniPCX Enteprise  ve SW verzí

R12.x v lokalitě Karlovy Vary a OmniPCX Office v záložní lokalitě Sokolov a na výjezdových základnách. Tyto systémy jsou propojeny za pomoci privátních SIP hlasových kanálů. V lokalitě Karlovy Vary je OMNIPCX Enteprise využívána pro příjem tísňových hovorů a pro administrativní provoz. Dále je na ni připojen systém AMDS, který slouží pro vyrozumění výjezdových skupin ZZS.

Popis budoucího stavu:

Bude proveden upgrade centrální PBX OmniPCX Enteprise na poslední dostupnou SW verzi R100.x včetně výměny HW komponentů pro zajištění bez výpadkového provozu. V lokalitě Sokolov bude umístěno redundantní řízení PBX a bude lokální PBX migrována do centrální PBX, výjezdové stanoviště Aš a Žlutice budou také migrovány do centrální PBX. V ostatních lokalitách výjezdových stanovišť bude provedena migrace TDM SW licencí na SW IP, včetně dodávky IP telefonu se zdrojem plně kompatibilní se systémem OmniPCX Enteprise. Centrální PBX bude doplněna o SW licence pro kontaktní centrum pro příjem tísňových hovorů. Pro integraci s aplikací SOS Dispečer bude dodána SW aplikace O2G s webovou službou v redundantním režimu. Pro automatickou obsluhu volajících do administrativní části ZZS bude dodána aplikace Visual automatická atendant s propojením na lokální DB. Aplikace bude instalována v redundantním režimu. Upgrade systému AMDS bude proveden systémem VNA (Visual notification asistant) pro vyrozumění výjezdových posádek, systém VNA bude integrován s dispečerskou aplikací webovou službou v redundantním režimu.

Název a počet požadovaných komponentů.

| Název SW a HW komponentů | Ks | Ks/Kč | Celkem |
| --- | --- | --- | --- |
| IP/SIP FOR FREE - DIGITAL USER REMOVED | 25 |  |  |
| IPMG - GD3/MR3-100/240V-150W | 1 |  |  |
| ISDN ACCESS T0 BRA8 BOARD | 1 |  |  |
| ISDN ACCESS E1 PRA-T2 BOARD | 1 |  |  |
| MIXED MIX4/4/8 BOARD | 2 |  |  |
| ANALOG INTERFACES SLI16-2 BOARD | 3 |  |  |
| INTERCONNECTION KIT POWERMEX-2/HSL1 | 1 |  |  |
| ARMADA DAUGHTER BOARD | 1 |  |  |
| DISK DRIVE FOR CPU8 / CS-3 | 2 |  |  |
| CALL SERVER 1GB BOARD OXE SW (CS-3) | 2 |  |  |
| GATEWAY DRIVER BOARD (GD-3) | 3 |  |  |
| BLIND SLOT STIFFENER | 5 |  |  |
| EXTERNAL RACK BATTERY BOX 36V | 1 |  |  |
| BATTERY 12V/7AH | 3 |  |  |
| GENERIC POWER CORD | 1 |  |  |
| OXE TELEPHONY REDUNDANCY - 1 USER | 89 |  |  |
| REMOTE EXTENSION PREMIUM - 1 USER | 100 |  |  |
| DISA SERVICE | 1 |  |  |
| IP PREMIUM DISCOUNTED - 1 USER - W/ SET | 25 |  |  |
| SIP NETWORK LINK BYPASS | 125 |  |  |
| ABC-F IP TRUNK GROUP ACCESS BYPASS | 2 |  |  |
| NETWORK ABC - 1 USER | 202 |  |  |
| SOFTWARE LICENSE OXE PURPLE R100.1 | 1 |  |  |
| OXE MAJOR UPGRADE - 1 USER | 80 |  |  |
| IPC/MIGRATION - SPS CONTRACT OXE | 352 |  |  |
| IP CENTRALIZATION | 1 |  |  |
| IP CENTRALIZATION OXO MIGRATION - 1 USER | 354 |  |  |
| VPS OR SIP LINK - 1 USER | 89 |  |  |
| CCD STARTER PACK 5 AGENTS | 1 |  |  |
| CCD 5 AGENTS PACK UP TO 50 | 1 |  |  |
| CCD AGENT REDUNDANCY - 1 AGENT | 10 |  |  |
| O2G VIRTUALIZATION | 1 |  |  |
| O2G CONTACT CENTER AGENT - 1 USER | 11 |  |  |
| O2G RELEASE R2.4 | 1 |  |  |
| O2G BASE | 1 |  |  |
| O2G ADVANCED TELEPHONY - 1 USER | 15 |  |  |
| O2G IP TOUCH XML - 1 USER | 15 |  |  |
| O2G HIGH AVAILABILITY BASE | 1 |  |  |
| O2G HIGH AVAILABILITY - 1 USER | 15 |  |  |
| ALE-20 DUAL GE IP DESKPHONE | 25 |  |  |
| JAVA RUNTIME ENV. SUPPORT FOR CCS/OXE | 1 |  |  |
| SUSE LINUX ENT SUPPORT FOR O2G | 2 |  |  |
| SUSE LINUX ENT SUPPORT FOR VAA | 2 |  |  |
| SUSE LINUX ENT SUPPORT FOR VNA | 1 |  |  |
| VAA - 1 PORT | 4 |  |  |
| VAA - 1 PORT HA | 4 |  |  |
| VAA - 1 PORT IVR | 4 |  |  |
| VAA SOFTWARE LICENSE R4.4 | 1 |  |  |
| VNA BROADCAST SERVICE - 1 USER | 100 |  |  |
| VNA NOTIFICATION AND WORKFLOW | 1 |  |  |
| VNA SOFTWARE LICENSE R2.2 | 1 |  |  |
| ENTERPRISE SPS 5 YEARS | 1 |  |  |
| VAA SPS 5 YEARS | 1 |  |  |
| VNA SPS 1 YEAR | 1 |  |  |
| O2G SPS 5 YEARS | 1 |  |  |
| Instalace, konfigurace, školení, doprava |  |  |  |